HERITAGE STATEMENT IN SUPPORT OF LISTED BUILDING APPLICATION

THE TREVOR ARMS, NO 1 MARFORD HILL, MARFORD, LL12 8TA

Coordinates :-

Latitude: 53.0996 / 53°5'58"N

Longitude: -2.9584 / 2°57'30"W

OS Eastings: 335924

OS Northings: 356246

OS Grid: SJ359562

Mapcode National: GBR 77.8MW8

Mapcode Global: WH88S.JCG5 *

Plus Code: 9C5V32XR+RJ

## History

The Trevor Arms is listed  as a C19 Inn and has  a Grade 2 I listing in the UK historic Buildings register. It is situated in the Marford Conservation area.

The building is an early C19  construction incorporating an earlier building. It is in the picturesque style of the group of cottages and houses built  in the ‘cottage orne’ style for the Trevalyn Estate under the stewardship of John Boydell. It was altered late C19 with the replacement of chimneys and extensions; interior remodelling C20. Including the introduction of a fireplace feature in a refurbishment circa 2013

The conservation area is centred on Marford village where the buildings all  have a distinct identity with ‘gothic’ style ogee windows.

.Exterior

The exterior is of Brick, render, slate roof with red ridge tiles and red brick chimneys. Rectangular plan with extensions to side and rear, 3-storey with 2-storey wings. Symmetrical elevation except at ground floor. Upper floors have 3 windows per floor all with ogee heads, those on the top floor of smaller dimensions; cill bands painted black. Ground floor has central entrance with later gabled porch. To right a large multipaned cast-iron window with ‘Gothic’ glazing detail and an ogee head, to left an arched entrance flanked by tall arched multipaned cast-iron windows. Wing to right has 1 cast-iron multipaned window with ‘Gothic’ glazing and ogee head on each floor. Wing to left has later door opening with ogee head and a window with ogee head in upper floor. Rear has various additions and retains a mock arrow slit and a tall arched window.

Interior

The building was constructed as an Inn and has had interior remodelling throughout its life

The Ground floor is divided into areas/ room sets  and has chamfered and stopped ceiling beams of C17 or earlier C18 character and areas of half timbered wall features. All timber detailing is currently painted finish in gloss black with windows in white gloss

It contains a vaulted ceiling dining area to one section

The windows have ogee heads and glazing panels and there are 2  internal doors with ogee heads

The floors are a mix of timber boarding , flagstones and carpet and do not appear to have a historic interest

The premises has contemporary fixtures and fittings commensurate with operating as a public house.

Proposals

The proposed scheme retains and respects the historic fabric of the building – and is intended to refurbish and maintain the building to a high standard.