MEMORANDUM

From: Ian Jones			To: Mathew Philips

Public Protection Service Manager			Head of Service Planning Development

Our Ref:				Your Ref: P/2023/0606

Date: 12-08-24

Town and Country Planning Act 1990

Town and Country Planning (General Development Procedure) Order 1996

Re: P/2023/0606, WYNN OFFA WYNNSTAY YARD RUABON WREXHAM

CAIS AMLINELLOL AR GYFER CODI ANNEDD (CADW POB MATER) / OUTLINE APPLICATION FOR ERECTION OF DWELLING (ALL MATTERS RESERVED)

- REPORT REVIEW

Report 'BS8233, BS4142 AND BS6472ASSESSMENT OF WYNN OFFA,WYNNSTAY YARD,RUABON, LL14 6DP' by E2 consultants dated March 18, 2024. Has considered the impart of noise from the road, railway line and a nearby industrial process. Vibration was also considered from the rail line.

For noise the report has identified that the site falls within Noise Exposure Category B of Technical Advice Note 11 and therefore mitigation is required. The report has considered mitigation in line with BS8233 for internal and external noise limits and proposed a scheme of double glazing and trickle ventilation for the habitable rooms in the proposal. The report has confirmed (paragraph 5.0.5) that the internal layout is not currently known and I recommend a suitably worded planning condition requiring the glazing, structural and ventilation recommendations as defined in chapter 6 be required. Further paragraph 9.0.5 has proposed a minimum standard for the boundary fencing so as to achieve the noise limits for external amenity areas. I recommend that this also be incorporated into the planning condition.

The report has considered the impact of vibration from passing trains and determined the levels to be significantly lower than the values where disturbance is likely and therefore no further investigation is warranted

The findings and mitigation methods detailed in the above report are accepted.

- INF22 – MINOR DEMOLITION CONSTRUCTION NOISE

All works in relation to the implementation of this permission, including deliveries to and /or leaving the site, should be undertaken only between the hours of 07.30 to 18.00 Monday to Friday, 08.00 to 14.00 on a Saturday, and at no time on a Sunday or a Bank Holiday. Outside these times, any works which are audible beyond the site boundary have the potential to cause unreasonable disturbance to neighbouring premises.

The applicant is advised that the Council has the option to control construction noise by serving a Control of Pollution Act 1974, Section 60 Notice where deemed necessary, and failure to comply with such a Notice can result in prosecution.

The applicant is further advised that any complaints received relating to noise of any type from the development may also be investigated using the Council’s Standardised Procedure for Dealing with Noise Nuisance Complaints. This is done under the Statutory Nuisance provisions provided within the Environmental Protection Act 1990, and legal action may be taken where appropriate.

The applicant must adhere to the times given above wherever possible. If, however, there are urgent or unavoidable circumstances requiring work outside of the specified times, permission to proceed may be sought by contacting Environmental Health & Housing Standards at healthandhousing@wrexham.gov.uk or 01978 292040. Please use the same contact details for any other enquiries regarding construction / demolition site noise.

- INF24 - DUST MANAGEMENT (SCHEME NOT REQUIRED BY CONDITION)

The Applicant is advised that under the dust from construction and / or demolition activities can be judged to be causing a statutory nuisance to neighbouring properties.  A legal notice can be served requiring that any dust nuisance is abated, and failure to comply with the requirements of the notice can result in prosecution.  The applicant should contact the Council’s Environmental Health & Housing Standards Team on healthandhousing@wrexham.gov.uk or 01978 292040 for further advice and information.

- INF23 - BURNING OF WASTE

Burning of waste generated from construction activities is not considered to be an appropriate method of disposal and action may be taken as follows:

-  Under the Environmental Protection Act 1990 anyone found   disposing of construction site waste by burning is likely to be in breach of their duty of care with regard to waste disposal;

- Under the same Act an abatement notice may be served where smoke is judged to be causing a nuisance to neighbouring properties. Failure to comply with the requirements of the notice can result in prosecution;

- Under the Clean Air Act 1993 it is an offence for a commercial activity to burn anything that gives rise to dark smoke.

To prevent offences under the above named Acts there should be no bonfires on the site, to include the prohibition of the burning of cleared vegetation. The applicant should contact the Council’s Environmental Health & Housing Standards Team on healthandhousing@wrexham.gov.uk or 01978 292040 for further advice and information.

p.p.

# Ian Jones

# Public Protection Service Manager