HERITAGE STATEMENT

ROYAL OAK WREXHAM

Proposal: LBC

Background

Joules has identified that the roof of the Royal Oak is in poor condition.  The slates themselves are in good condition, however many have slipped and are now unsecure.  The property has water ingress which over time will threaten the wellbeing of the fabric of the building.

LISTING

The building is grade II listed and as such requires an LBC in order to re roof the building.  Advice was sought if this would be required, and the conservation officer confirmed the same.

The listing is registered as The Embassy Public House’

The listing detail is as follows:

Grade

II

Status

Designated

Date of Designation

31/01/1994

Date of Amendment

31/01/1994

Name of Property

Embassy Public House

Public House. Built c1912-13, replacing an earlier timber framed building.

Exterior

Mock timberwork with roughcast panel infill, and red sandstone lower storey. 3 storeys, single window range, with gable facing street. Projecting canted entrance porch at right hand corner, with mullioned overlight and sidelights to doorway across angle. Continuous fenestration, comprising 3 paired mullioned lights with leading to the left. Oriel window to first floor, and jettied upper storey with 3-light casement window. Fretted bargeboards with finial to gable.

Rationale For the Works

Re roofing is required periodically in order to preserve the integrity of the building.  This is good practice before water ingress affects the fabric of the building.

Detailed Description of the works.

The applicant has consulted with the conservation  officer both through e mail correspondence and also on site.  The proposal is to re move the slate tiles, and decayed batons.  The roof rafters will be inspected and are believed to be in good condition.  A layer of breathable roof membrane shall be applied, the roof re batons re placed and fixed and the existing slate re applied.

It is usual to expect around 20% losses which will bee made up with sourced Welsh slate.  The top up slates have already been approved by the conservation officer.

Benefits of the works to the heritage asset.

One the core reasons for a buildings decay is water ingress.  In this case the listed asset has had a number of issues over the last few years with damage to the internal ceilings and walls of the floor rooms.  The issue has been both the decayed slate roof and compromised valleys.  The roof will need to be restored within the next few years or more Significant water ingress can be expected, this will in the medium term threaten the condition of the building.  In the event that this work is not carried out then we are advised, over time, the decay to the fabric will cause deterioration and could even threaten the viable use of the building requiring much more expensive investment and the possibility of more invasive work to the heritage asset being required to maintain it for the future.

For this reason, in order to preserve the fabric of the building then it has been very strongly recommended that the re-roofing is carried out in a timely manner and this is essential long term good maintenance of the listed building.

With the method and the materials all now agreed by the conservation  officer then listed building approval is sort to formally recognise the works.