MEMORANDUM

From: Ian Jones			To: Mathew Philips

Public Protection Service Manager			Head of Service Planning Development

Our Ref:				Your Ref: P/2024/0319

Date: 04-10-24

Town and Country Planning Act 1990

Town and Country Planning (General Development Procedure) Order 1996

Re: P/2024/0319, THE COACH HOUSE ASHGROVE CHESTER ROAD GRESFORD WREXHAM

ESTYNIAD DEULAWR A NEWID DEFNYDD Y LLAWR GWAELOD O GAREJ I SIOP GOFFI / TWO STOREY EXTENSION AND CHANGE OF USE OF GROUND FLOOR FROM GARAGE TO COFFEE SHOP

RPT1 - REPORT REVIEW

Report 'Residential Noise Assessment, Site Address: The Coach House, Ashgrove, Chester Road, Wrexham, LL12 8PP' Dated 12-09-24, Ref: NP-011504 by Nova Acoustics. Has considered the exiting noise environment for the proposed residential accommodation where monitoring showed that the daytime noise levels were at the crossover between Noise Exposure Category (NEC) A and B and night time noise levels in NEC B of Technical Advice Note 11, indicating that mitigation is required. The assessment point closest to the care home showed levels within NEC B. The report has provided a glazing and background ventilation specification as a mitigation scheme.

The report has further considered noise breakthrough from the commercial ground floor use to the first floor accommodation. The report has justified slightly reducing the nose insulation requirement  of the floor on the basis that the property will be occupied by the person responsible for managing the commercial property who will therefore have control over operations as well as background music for example. The commercial use was also confirmed not be operating at night. This is accepted on the basis that planning conditions restrict occupancy to staff of the commercial use and that opening times are restricted to 09:00 to 21:00 Mon to Sun as stated in paragraph 1.2 of the report. The report has provided an upgrade scheme in paragraph 5.2. and confirmed that sound insulation testing will be required upon completion of any work.

The report has also considered the impact from plant associated with both the care home and the proposed use. Existing plant has been mitigated against with the proposed glazing specification. Planned plant has been confirmed to be of sound levels that will not affect residential amenity though the report has confirmed that good acoustic practice should be followed in their installation, the plant terminations should be positioned away from the above bedroom window regardless of the low noise emissions expected. All plant should be regularly serviced and maintained to ensure it is operating at optimal efficiency.

The findings and recommendations of the report are accepted and I recommend the mitigation methods proposed in the report be conditioned as detailed above.

………………………………………………

Head of Service Public Protection