Access statement for Land south of 27 to 30 Trofryn Penycae

The site is located off an unadopted highway; vehicular speeds are unlikely to be any more than 20 mph.  Welsh Government guidance recommends a splay of 2m x 25m in both directions for the presumed speed of traffic; it is possible to achieve these splays from the proposed car park on to the unadopted road.  There are a number of small self seeded trees alongside the road which will be removed to ensure full visibility is achievable from the car park.

Visibility from the unadopted road on to the classified highway known as Afoneitha Road is concurrent with Welsh Government recommendations for a 20mph speed limit.

The unadopted road is owned and maintained by Wrexham County Borough Council Housing Department.  It has been agreed that the unadopted road will be resurfaced in its entirety from Afoneitha Road to the car park near Penycae Football Club; meaning that access to the proposed car park will be taken along an adequately surfaced and properly maintained highway.

There is therefore no reason for the highway authority to object to this application.