Site Services Review – Annual SUD’s Maintenance Plan Woodlands

Date: 20/01/2022

All maintenance will be carried out in line with the RSL’s asset management annual maintenance plan and will be reviewed annually.

The soakaways are located within individual residents gardens and they will maintain responsibility of regular/routine maintenance of the garden as shown in the below maintenance plan. However, property conditions are inspected throughout the year, to allow the association to ensure maintenance and upkeep of properties.

The management plan will commence 12 months post completion, where the RSL takes full ownership of maintenance and management of the scheme.

| Maintenance Type | Frequency | Task | Responsible Persons |
| --- | --- | --- | --- |
| General Maintenance | Weekly/ Monthly basis dependant on season | - Grass cutting and general garden maintenance - Checking of any outlets and reporting any issues to WWHA | Resident |
| Occasional | 6-12 months | - Inspection of soakaway - Weather monitoring after inspection after heavy rain - Inspection for Tree roots - Inspection of surface settlement - Inspection of inlets and outlets and pipework - Cleaning of roof gutters - Sediment Removal | WWHA |
| Remedial Maintenance | When Required | - Any repairs to structure and surface reconditioning | WWHA |