Access statement for 33 Nant y Gaer Road Llay

The site is located on a classified highway within a 30 mph speed limit although vehicular speed is considered to be no more than 25 mph due to the road geometry.  Welsh Government guidance recommends a splay of 2m x 33m in both directions for the presumed speed of traffic; it is not possible to achieve these splays from the proposed access.

However if the tenant has no off road parking facility then their vehicle will be parked on the inside of a bend, which will force passing cars into the opposite carriageway and into oncoming traffic, which is potentially a dangerous situation particularly on a bend.

Visibility at the proposed access will be maximised by setting back existing boundary fences and in my opinion the off road parking facility will result in a safer car parking facility than leaving the car on the street.

Due to the size of the plot, it is not feasible to provide an off road turning facility within the curtilage of the site. However, there are numerous accesses in the immediate vicinity which do not provide turning facility; and indeed many of these don’t even have a formalised vehicular crossing.

It is therefore my opinion that the highway authority should be willing to support application to create a new access at this location