Heritage Impact statement Croxton Farmhouse

| Proposed Works | Objective | Significance of affected fabric | Assessing beneficial impact | Assessing Harmful Impact | Proposed solution |
| --- | --- | --- | --- | --- | --- |
| Attic – make good not being used Wood Worm treatment Make good all walls using lime plaster Make good doors – repaint Paint all walls Making good floor boards where possible like for like Over board floor board to protect Fire sprinkler system to be installed FD 30 fire door to go at the bottom of the stairs – no door at present | To make good a severely run down & un safe section of the house | The attic forms part of the original building | Ensures the building is well maintained Walls will be brought back to original standard | Possibly some loss of original floor board material due to wood worm present | Design the works to minimise the impact and to ensure best match for new floor boards |
| Formation of en- suites | To provide suitable bathroom facilities for new occupants | External walls – waste and soil pipes, extraction units Internal wall cabling for electric shower Ceiling – new light | Allows the building to have a more viable use | External brick work Ceilings where the new lights will be located which can be reversed | Works have been planned to be fixed to current pipes Check if boiler - water pressure is good enough for Barr Shower |
| Installation of L1 fire system | This will be a remote wireless system to protect life this is a requirement with all external regulatory bodies | No material change other than chasing in of electrical wire in managers office for main panel | Saves lives – will not impact on original floor boards & ceilings to the home | Chasing in the main electrical cable from the mains panel which will be located in the manager office | This is the best solution as the other option is a hardwired one and would create more harm to the floor boards & ceilings |
| Installation of fire sprinkler system | To comply with building control regulations | All floor boards Ceilings | To protect life | Up lifting of floor boards, ceilings where sprinkler heads are to be located - this system can be taken out and all works reversed | Works have been designed to minimise as much impact as possible |
| Replacing Internal doors for FD 30 | To comply with Fire safety | Original internal doors – possibly 6 | To protect life and can be reversed with no impact to the original doors in place | Possibly 3 original internal doors will have to be changed | Fire risk assessment carried out and with the Fire sprinkler system and also the L1 remote fire system going in this should be enough Original doors are thick enough to possibly be left in place The company have had a quote for a like for like fire door |
| Replace Door furniture | To ensure that all doors have a master suited lock system that the young people are not able to have access too – Knobs will be fitted | Original doors | To keep the young people safe and a requirement of CIW – Knobs will ensure that the young people are not able to barricade there selves in . | Doors will have to have holes made to fit the barrel & knobs added | We can get like for like doors made and take the original ones off and store |
| Managers office | To have a separate office for the day to day care workers to enable them able to have a confidential work space | Main dividing wall – which is rotten | The wall is currently rotten and has not been maintained – so any works will benefit this area | The wall will have to be taken down and rebuilt using a stud wall | Treat the wall and then stud over it |
| Young person’s bedrooms x 2 | To provide suitable living accommodation for young people in care to have a personal space | Fire place to be boarded up , door frame & original door | The fire place will be protected Fire door will protect life | The fire place will not be on show however the barding up will be reversible | Leave the fire place open Replace the door with a like for like fire door |
| Drainage works | Excavate a new foul drain and attach to existing man hole to the rear of the property, to create the new en- suites | Brick work – floor boards | The home will benefit from new en - suites | Brick work will have to have some work’s carried out | Chase the pipes under the floor boards which will mean removing more that would not need to be touched if the new drainage was put in |
| Staff Bedrooms x 2 | Protect the fire place | Nothing | The fire lace will be protected | None | Leave fire place open |