For the Attention of: Mared Rees-Jones – Case Officer

Wrexham County Borough Council

[By Email: planning@wrexham.gov.uk]

9 November 2020

Dear Mared

PLANNING APPLICATION: P/2020/0647

Erection of detached dwelling at Y Bryn, Duke Street, Rhosllanerchrugog, Wrexham

Thank you for your consultation letter of 21 October 2020 seeking the views of the Coal Authority on the above planning application.

The Coal Authority is a non-departmental public body sponsored by the Department of Business, Energy & Industrial Strategy. As a statutory consultee, the Coal Authority has a duty to respond to planning applications and development plans in order to protect the public and the environment in mining areas.

The Coal Authority Response: Substantive Concern

The application site falls within the defined Development High Risk Area; therefore within the site and surrounding area there are coal mining features and hazards which need to be considered in relation to the determination of this planning application.

The Coal Authority records indicate that the potential zones of influence/instability associated with two off-site mine entries (shafts, CA refs. 329346-426 and 329346-427) encroach into the application site. We hold no details of any past treatment of these coal mining related features and, due to potential plotting inaccuracies, the actual positions of the shaft could deviate from their plotted positions by several metres.

In addition, our information indicates that the site lies in an area where underground coal mining is recorded to have taken place at shallow depth and where further historic unrecorded shallow coal mining is likely to have taken place.

The applicant has submitted a Coal Mining Risk Assessment Report (13 October 2020, prepared by EnviroSolution Ltd) to accompany their planning application; however, the Coal Authority does not consider that this adequately addresses the impact of coal mining legacy on the proposed development. We therefore OBJECT to the application.

The submitted report draws upon relevant sources of coal mining and geological information and concludes that possible shallow coal mine workings pose a potential stability risk to the proposed development. We therefore welcome the recommendation for the undertaking of intrusive site investigation (boreholes) in order to ascertain ground conditions and the depth and condition of shallow coal seams/workings. Our concerns, however, relate to the assessment of the risk posed by the recorded off-site mine entries to the proposed development.

We take this opportunity to highlight that any form of development over or within the influencing distance of a mine entry can be dangerous and raises significant safety and engineering risks and exposes all parties to potential financial liabilities. The Coal Authority has adopted a policy where, as a general precautionary principle, the building over or within the influencing distance of a mine entry should wherever possible be avoided. Our adopted policy on the matter can be found at:

www.gov.uk/government/publications/building-on-or-within-the-influencing-distance-of-mine-entries

Section 3 of the report acknowledges the presence of the two recorded mine entries in the vicinity of the site. It goes on to state that “It is noted that the current best-plot-position of the shafts (distance from site boundary) is greater than the estimated departure, ie. the shafts almost certainly fall outside the site boundary. However, they could still influence the positions of built development within the site boundary.” In respect of the shafts, Section 4 of the report advises that “It is considered that intrusive investigations to determine the thickness of the superficial cover may reduce the risk of potentially unstable ground affecting the site.”

The Coal Authority concurs that the shafts are unlikely to be present within the application site, however, they could potentially be present very close to, or even immediately adjacent to, the site boundary. We also agree that investigations into the depth of superficial deposits would assist in the calculation of the extent of the potential zones of influence in the event of an off-site shaft collapse. However these investigations have yet to be carried out and the information presented in support of the planning application does not demonstrate that the proposed development, for which detailed permission is being sought, would be sited clear of the zones of influence in the event of an off-site shaft collapse.

In light of the above, the Coal Authority considers that further specialist investigation is required prior to the determination of the application:

- to establish the potential zones of influences for the off-site shafts;

- to define appropriate ‘no build’ zones around the shafts; and

- to demonstrate to the LPA through the submission of a revised proposed Site Plan that the site can be safely developed whilst avoiding the defined ‘no build’ zones. This may necessitate the resiting of the proposed building.

The above information, along with proposed remedial, preventive and mitigatory measures, should be included as part of a revised Coal Mining Risk Assessment report to address coal mining legacy related safety and stability issues affecting the site.

The applicant is requested to note that Permission is required from the Coal Authority Permit and Licensing Team before undertaking any activity, such as ground investigation and ground works, which may disturb Coal Authority property, i.e. coal seams and associated workings.

The Coal Authority Recommendation to the LPA

The Coal Mining Risk Assessment Report has revealed that there is a significant risk to development at the application site, and it is the opinion of the Coal Authority that the risk and uncertainty is such that further specialist investigation is required before the LPA can be satisfied that the application site can be safely developed.

The applicant should therefore be advised of The Coal Authority’s view as technical expert in the field of coal mining legacy and land instability.  Furthermore, they should be required to carry out intrusive site investigation works, subject to obtaining Coal Authority permission, to determine the extent of past coal mining activities and the implications for the layout and design for their development proposal. Any remedial, preventive and mitigation measures should then be proposed as part of the revised Coal Mining Risk Assessment Report, as necessary, to address any issues of land instability.

The Coal Authority would be pleased to receive for further consultation and comment any additional information prepared and submitted by the applicant.

Please do not hesitate to contact me if you would like to discuss this matter further.

Yours sincerely

James Smith

James Smith BSc. (Hons), Dip.URP, MRTPI

Planning Liaison Manager

General Information for the Applicant

Under the Coal Industry Act 1994 any intrusive activities, including initial site investigation boreholes, and/or any subsequent treatment of coal mine workings/coal mine entries for ground stability purposes require the prior written permission of The Coal Authority, since such activities can have serious public health and safety implications. Failure to obtain permission will result in trespass, with the potential for court action. In the event that you are proposing to undertake such work in the Forest of Dean local authority area our permission may not be required; it is recommended that you check with us prior to commencing any works. Application forms for Coal Authority permission and further guidance can be obtained from The Coal Authority’s website at:

www.gov.uk/get-a-permit-to-deal-with-a-coal-mine-on-your-property

Disclaimer

The above consultation response is provided by The Coal Authority as a Statutory Consultee and is based upon the latest available data on the date of the response, and electronic consultation records held by The Coal Authority since 1 April 2013.  The comments made are also based upon only the information provided to The Coal Authority by the Local Planning Authority and/or has been published on the Council's website for consultation purposes in relation to this specific planning application.  The views and conclusions contained in this response may be subject to review and amendment by The Coal Authority if additional or new data/information (such as a revised Coal Mining Risk Assessment) is provided by the Local Planning Authority or the Applicant for consultation purposes.

In formulating this response The Coal Authority has taken full account of the professional conclusions reached by the competent person who has prepared the Coal Mining Risk Assessment or other similar report.  In the event that any future claim for liability arises in relation to this development The Coal Authority will take full account of the views, conclusions and mitigation previously expressed by the professional advisers for this development in relation to ground conditions and the acceptability of development.