For the Attention of: Matthew Phillips

Wrexham County Borough Council

[By Email: planning@wrexham.gov.uk]

01 July 2020

Dear Mr Phillips

PLANNING APPLICATION: P/2019/0122 RECONSULTATION

Outline application for the erection of 1 no. detached dwelling (with all matters reserved); Land Rear Of, Hillcrest North Of, Bethania Road, Acrefair, Wrexham, LL14 3UD

Thank you for your consultation letter of 30 June 2020 seeking the views of The Coal Authority on the above planning application.

The Coal Authority is a non-departmental public body sponsored by the Department of Business, Energy & Industrial Strategy.  As a statutory consultee, The Coal Authority has a duty to respond to planning applications and development plans in order to protect the public and the environment in mining areas.

The Coal Authority Response: Material Consideration

I have reviewed the proposals and confirm that the application site falls within the defined Development High Risk Area.

The Coal Authority records indicate that within the application site and surrounding area there are coal mining features and hazards which need to be considered in relation to the determination of this planning application, specifically The Coal Authority records indicate that the site is underlain by recorded shallow coal workings and probable unrecorded underground shallow coal workings.  In addition, the records indicate that the application site is likely to have been subject to historic unrecorded coal mine workings at shallow depth associated with a thick coal outcrop and the site also lies within a Surface Coal Resource Zone.

The Coal Authority is now pleased to note that the applicant has obtained appropriate and up-to-date coal mining information for the proposed development site.  This information has been used to inform a Coal Mining Risk Assessment (or equivalent) (June 2020, prepared by Groundsolve Limited) to accompany the planning application.

The report is able to recommend that two to three rotary percussive boreholes are drilled beneath the building footprint to determine the depth and condition of any shallow mine workings beneath the site.  If workings are encountered in the Quaker coal seam, the report recommends that it will be necessary to drill and grout the workings to stabilise the ground beneath the site.

The report advances to identify that in the event no workings are identified in the Quaker seam, with workings in the deeper seams then raft foundations designed to span any potential drown holes up to 3m in diameter and cantilever 1.5m at its edge should be sufficient to address the low risk.  This aspect would be a matter to be controlled via the Building Regulations process.

The Coal Authority considers that due consideration should also be afforded to the potential risk posed by mine gas to the proposed development.

The applicant should ensure that the exact form of any intrusive site investigation, including the number, location and depth of boreholes, is agreed with The Coal Authority’s Permitting Team as part of their permit application.

The Coal Authority Recommendation to the LPA

The Coal Authority concurs with the recommendations of the Coal Mining Risk Assessment Report; that coal mining legacy potentially poses a risk to the proposed development and that intrusive site investigation works should be undertaken prior to development in order to establish the exact situation regarding coal mining legacy issues on the site.

Accordingly, the Coal Authority recommends the imposition of the following conditions:

- No development shall commence until intrusive site investigations have been carried out on site to establish the exact situation in respect of coal mining legacy features.  The findings of the intrusive site investigations shall be submitted to the Local Planning Authority for consideration and approval in writing.   The intrusive site investigations shall be carried out in accordance with authoritative UK guidance.

- Where the findings of the intrusive site investigations (required by condition XX above) identify that coal mining legacy on the site poses a risk to surface stability, no development shall commence until a detailed remediation scheme to protect the development from the effects of such land instability has been submitted to the Local Planning Authority for consideration and approval in writing.  Following approval, the remedial works shall be implemented on site in complete accordance with the approved details.

The Coal Authority therefore has no objection to the proposed development subject to the imposition of the conditions to secure the above.

This is our recommendation for condition wording. Whilst we appreciate that you may wish to make some amendment to the choice of words, we would respectfully request that the specific parameters to be satisfied are not altered by any changes that may be made.

Please do not hesitate to contact me if you would like to discuss this matter further.

Yours sincerely

Ashley Langrick

Ashley Langrick BA (HONS) Dip TRP MRTPI 
Planning and Development Manager (Corporate Projects)

General Information for the Applicant

Under the Coal Industry Act 1994 any intrusive activities, including initial site investigation boreholes, and/or any subsequent treatment of coal mine workings/coal mine entries for ground stability purposes require the prior written permission of The Coal Authority, since such activities can have serious public health and safety implications.  Failure to obtain permission will result in trespass, with the potential for court action.  In the event that you are proposing to undertake such work in the Forest of Dean local authority area our permission may not be required; it is recommended that you check with us prior to commencing any works.  Application forms for Coal Authority permission and further guidance can be obtained from The Coal Authority’s website at:

https://www.gov.uk/get-a-permit-to-deal-with-a-coal-mine-on-your-property

Disclaimer

The above consultation response is provided by The Coal Authority as a Statutory Consultee and is based upon the latest available data on the date of the response, and electronic consultation records held by The Coal Authority since 1 April 2013.  The comments made are also based upon only the information provided to The Coal Authority by the Local Planning Authority and/or has been published on the Council's website for consultation purposes in relation to this specific planning application.  The views and conclusions contained in this response may be subject to review and amendment by The Coal Authority if additional or new data/information (such as a revised Coal Mining Risk Assessment) is provided by the Local Planning Authority or the Applicant for consultation purposes.

In formulating this response The Coal Authority has taken full account of the professional conclusions reached by the competent person who has prepared the Coal Mining Risk Assessment or other similar report.  In the event that any future claim for liability arises in relation to this development The Coal Authority will take full account of the views, conclusions and mitigation previously expressed by the professional advisers for this development in relation to ground conditions and the acceptability of development.