For the Attention of: Sharon Holman – Case Officer

Wrexham County Borough Council

[By Email: planning@wrexham.gov.uk]

17 December 2019

Dear Sharon Holman

PLANNING APPLICATION: P/2019/0894

ERECTION OF 3 NO DETACHED TWO STOREY DWELLINGS; Land North Of 8, Grango Lane, Ponciau, Wrexham, LL14 1ER

Thank you for your consultation letter of 09 December 2019 seeking the views of The Coal Authority on the above planning application.

The Coal Authority is a non-departmental public body sponsored by the Department of Business, Energy & Industrial Strategy.  As a statutory consultee, The Coal Authority has a duty to respond to planning applications and development plans in order to protect the public and the environment in mining areas.

The Coal Authority Response: Fundamental Concern

I have reviewed the proposals and confirm that the application site falls within the defined Development High Risk Area; therefore within the application site and surrounding area there are coal mining features and hazards which need to be considered in relation to the determination of this planning application.

The Coal Authority records indicate that the application site lies in an area of probable unrecorded coal mine workings at shallow depth and thick coal outcrops. In addition, our records indicate the presence of mine entry 329346-061 conjectured to be on the western application site boundary. This feature is subject to a departure of several metres from its recorded position and may be within the application site. The Coal Authority is of the opinion that building over the top of, or in close proximity to, mine entries should be avoided wherever possible, even after they have been capped, in line with our adopted policy:

https://www.gov.uk/government/publications/building-on-or-within-the-influencing-distance-of-mine-entries

The applicant has submitted a ‘Plots 5 to 7’ Mining Study (Ground Solve Ltd, 3 December 2019) which is informed by a range of mining information for the application site and proposed development, including the findings of intrusive site investigations from the neighbouring site.

The report author identifies that the application site will need to be drilled and grouted in order to stabilise underlying shallow coal mine workings. The Coal Authority would usually expect an appropriate scheme of intrusive site investigations first, however, in this particular case the author has clearly identified the need for remediation. On the basis of the information submitted we consider it necessary that the applicant undertakes the recommended scheme of drilling and grouting.

The report has also discussed intrusive site investigations undertaken to locate a recorded mine entry immediately west of plot 4 which is not within the site boundary. It is unclear what mine entry the report author is discussing; but based upon the ‘suspected shaft’ annotation on the Location of Drilling and Grouting Plots 1 to 3 drawing, in the submitted report, it appears the report author is referencing mine entry 329346-060. I note that this mine entry 329346-060 is conjectured to be beyond influencing distance of the application site and the proposed development. Unfortunately, it seems the report has failed to identify and discuss the presence of mine entry 329346-061 on the immediate western site boundary.

I have reviewed our records for mine entry 329346-061 against the Site & Location Plan (Drawing no. P/03/A, 15 October 2019). This plan plots the probable position of the mine entry immediately south of plot 6, however, the associated zone of influence has not been plotted. As such the relationship between proposed built development and the mine entry has not robustly been demonstrated. It is the opinion of The Coal Authority that the built development may potentially be proposed in close proximity to mine entry 329346-061 and within its zone of influence. Therefore, The Coal Authority objects to this planning application in accordance with our aforementioned policy on mine entries.

The applicant should be advised that we expect intrusive site investigations to be undertaken within the application site, prior to determination, in an attempt to locate the mine entry and determine its condition. It would be advisable for the applicant to seek the geo-referencing of this feature prior to the intrusive site investigations being designed. Findings from these investigations should be used to identify an appropriate ‘no-build’ zone around the mine entry, taking into account depth to rock head and the radius of the entry etc. Revisions to the site layout should then be made in order to ensure that built development avoids the ‘no-build’ zone which should be clearly defined on the site layout plan. This revised layout plan should then be submitted for consideration along with any remediation and mitigatory measures proposed.  An appropriate scheme of remedial and mitigatory measures should also be proposed and submitted for approval prior to determination.

If after adequate investigations evidence is provided to demonstrate that the mine entry is off-site The Coal Authority would expect site specific data from site investigations to inform a worst-case zone of influence for the mine entry. This should include the best plotted position, departure and depth to rock head etc. This information should be used to consider the potential risks posed by an off-site collapse. Suitable measures need to be proposed to mitigate the impact of such an event and these should be submitted for approval. Information should also be provided to demonstrate that the siting of buildings to outside of the identified areas of risk has been properly considered. We will still expect the submission of a layout plan plotting the mine entry and its zone of influence along with justification for all measures proposed.

The exact form and extent of intrusive site investigations need to be agreed with the Permitting Section of The Coal Authority as part of the applicant’s permit application. These intrusive site investigations should be prepared and conducted by a suitably competent person and findings used to inform an appropriate scheme of remedial measures if necessary. In addition, it would be prudent if consideration was also afforded to the risk posed by mine gas to the application site and proposed development.

The Coal Authority Recommendation to the LPA

The Coal Authority welcomes the precautionary approach recommended by the author of the report for phases of remediation and investigation. However, it is the opinion of The Coal Authority that the risk and uncertainty is such that further specialist investigation is required before the LPA can be satisfied that the application site can be safely developed.

The applicant should therefore be advised of The Coal Authority’s view and should be required to carry out intrusive site investigation works, subject to obtaining Coal Authority permission, to determine the extent of past coal mining activities and the implications for the layout and design for their development proposal. Any remedial, preventive and mitigation measures, as necessary, should then be proposed along with a revised layout plan which has clearly defined ‘no-build’ zones.

Any investigation and remediation works need to be agreed with the Permitting Section of The Coal Authority and should be prepared and conducted by a suitably competent person.

The Coal Authority would be very pleased to receive for further consultation and comment any additional information prepared and submitted by the applicant.

Please do not hesitate to contact me if you would like to discuss this matter further.

Yours Sincerely,

George Weightman

George Weightman | MSc. BSc.

Assistant Planning Liaison Manager

General Information for the Applicant

Where development is proposed over areas of coal and past coal workings at shallow depth, The Coal Authority is of the opinion that applicants should consider wherever possible removing the remnant shallow coal.  This will enable the land to be stabilised and treated by a more sustainable method; rather than by attempting to grout fill any voids and consequently unnecessarily sterilising the nation’s asset.  Prior extraction of surface coal requires an Incidental Coal Agreement from The Coal Authority.  Further information can be found at:

https://www.gov.uk/get-a-licence-for-coal-mining

Under the Coal Industry Act 1994 any intrusive activities, including initial site investigation boreholes, and/or any subsequent treatment of coal mine workings/coal mine entries for ground stability purposes require the prior written permission of The Coal Authority, since such activities can have serious public health and safety implications.  Failure to obtain permission will result in trespass, with the potential for court action.  In the event that you are proposing to undertake such work in the Forest of Dean local authority area our permission may not be required; it is recommended that you check with us prior to commencing any works.  Application forms for Coal Authority permission and further guidance can be obtained from The Coal Authority’s website at:

https://www.gov.uk/get-a-permit-to-deal-with-a-coal-mine-on-your-property

Disclaimer

The above consultation response is provided by The Coal Authority as a Statutory Consultee and is based upon the latest available data on the date of the response, and electronic consultation records held by The Coal Authority since 1 April 2013.  The comments made are also based upon only the information provided to The Coal Authority by the Local Planning Authority and/or has been published on the Council's website for consultation purposes in relation to this specific planning application.  The views and conclusions contained in this response may be subject to review and amendment by The Coal Authority if additional or new data/information (such as a revised Coal Mining Risk Assessment) is provided by the Local Planning Authority or the Applicant for consultation purposes.

In formulating this response The Coal Authority has taken full account of the professional conclusions reached by the competent person who has prepared the Coal Mining Risk Assessment or other similar report.  In the event that any future claim for liability arises in relation to this development The Coal Authority will take full account of the views, conclusions and mitigation previously expressed by the professional advisers for this development in relation to ground conditions and the acceptability of development.