For the Attention of: Mr Matthew Phillips – Case Officer

Wrexham County Borough Council

[By Email: planning@wrexham.gov.uk]

[Copy Email: Matthew.Phillips@wrexham.gov.uk]

12 June 2019

Dear Mr Phillips

PLANNING APPLICATION: P/2018/1047

Demolition of existing property and redevelopment to form 2 no new dwellings and access; Nantcol, 57 Bryn Glas, Rhosllanerchrugog, Wrexham, LL14 2EA: Amended Plans / Description

Thank you for your notification of 23 May 2019 seeking the views of the Coal Authority on the amended plan Drawing No. C063/022, October 2018) and amended description of development.  It is noted that the above plan now annotates that the area where the Coal Authority records identify the position of a mine entry, is not part of this current application.

The Coal Authority Response: Material Consideration

Taking into consideration the above, and that the proposed development is only for the formation of 2no. new dwellings, this proposal has now been informed by the presence of the mine entry. As per our letter dated 20 December 2018, our preferred option would be for the applicant to amend the layout to ensure that built development avoids the mine shafts (including any known deviation from the plotted position together with their resultant zone of influence).  Accordingly, if the applicant is able to amend the layout, we consider that the proposed mitigation strategy as identified within the Mining Study (Section 8) would be appropriate and acceptable.

Whilst no development is likely to be taking place over the recorded mine entry it should be noted that one of the plots could still be within influencing distance of this mine shaft (in the event of catastrophic failure).

However as recommendations have been made that intrusive site investigation are required in order to establish the exact ground conditions beneath the site, the findings of these investigations will also enable the applicant’s technical consultants to establish the depth to rockhead, so that a calculated zone of influence from the mine entry can be confirmed (in the event of a collapse). Should this zone of influence extend into the site, the applicant should be required to identify a “no build” zone and / or robust mitigatory measures to ensure the safety and stability of the development.

In light of the above, the Coal Authority is able to withdraw its objection to this proposal, subject to the imposition of a planning condition, by the LPA to secure the site investigations as per recommended by the applicant’s technical consultants, GroundSolve Ltd.

A condition should therefore require prior to the commencement of development:

* The undertaking of an appropriate scheme of intrusive site investigations;

* The submission of a report of findings arising from the intrusive site investigations;

* The submission of a scheme of remedial works for approval including a plan which identifies the calculated zone of influence of the mine entry; and

* Implementation of those remedial works.

Please do not hesitate to contact me if you would like to discuss this matter further.

Yours sincerely

D Roberts

Deb Roberts M.Sc. MRPTI
Planning Liaison Manager

General Information for the Applicant

Under the Coal Industry Act 1994 any intrusive activities, including initial site investigation boreholes, and/or any subsequent treatment of coal mine workings/coal mine entries for ground stability purposes require the prior written permission of the Coal Authority, since such activities can have serious public health and safety implications.  Failure to obtain permission will result in trespass, with the potential for court action.  In the event that you are proposing to undertake such work in the Forest of Dean local authority area our permission may not be required; it is recommended that you check with us prior to commencing any works.  Application forms for Coal Authority permission and further guidance can be obtained from the Coal Authority’s website at:

https://www.gov.uk/get-a-permit-to-deal-with-a-coal-mine-on-your-property

Any form of development over or within the influencing distance of a mine entry can be dangerous and raises significant safety and engineering risks and exposes all parties to potential financial liabilities.  As a general precautionary principle, the Coal Authority considers that the building over or within the influencing distance of a mine entry should wherever possible be avoided.  In exceptional circumstance where this is unavoidable, expert advice must be sought to ensure that a suitable engineering design is developed and agreed with regulatory bodies which takes into account of all the relevant safety and environmental risk factors, including gas and mine-water.  Your attention is drawn to the Coal Authority Policy in relation to new development and mine entries available at: https://www.gov.uk/government/publications/building-on-or-within-the-influencing-distance-of-mine-entries

Disclaimer

The above consultation response is provided by The Coal Authority as a Statutory Consultee and is based upon the latest available data on the date of the response, and electronic consultation records held by The Coal Authority since 1 April 2013.  The comments made are also based upon only the information provided to The Coal Authority by the Local Planning Authority and/or has been published on the Council's website for consultation purposes in relation to this specific planning application.  The views and conclusions contained in this response may be subject to review and amendment by The Coal Authority if additional or new data/information (such as a revised Coal Mining Risk Assessment) is provided by the Local Planning Authority or the Applicant for consultation purposes.

In formulating this response The Coal Authority has taken full account of the professional conclusions reached by the competent person who has prepared the Coal Mining Risk Assessment or other similar report.  In the event that any future claim for liability arises in relation to this development The Coal Authority will take full account of the views, conclusions and mitigation previously expressed by the professional advisers for this development in relation to ground conditions and the acceptability of development.