- Discharge of conditions – I can find no evidence that condition 11 of P/2015/0463 has been discharged.  Conditions 9 (referring to compliance with the Habitat Management Plan and Protected Species Method Statement) and 10 (hedgerow planting) have also not been complied with.

- Habitat Management Plan (15th October 2015)

- The current biodiversity value of the proposed mitigation site has actually decreased from that shown in the phase 1 habitat plan (figure 1) of the approved Habitat Management Plan.  It appears that some of the semi natural vegetation has been removed and some wet hollows within the floodplain have been filled in and levelled out see photograph 1. This is an enforcement matter for NRW and WCBC as constitutes both an engineering operation and engineering works within a flood plain.

- 3.1.1 – There is no evidence that the specified seed mix was used, the grassland beneath the solar panels appears to be rank and short improved grassland with limited floristic diversity (photograph 2).

- 3.1.2 – The 12m corridor is either absent or not fenced correctly.

- 3.1.3 – No evidence of any hedgerow improvements.

- 3.1.4 – No refugia piles were observed.

- 3.2.1 – Far from receiving a traditional hay cut the grassland within the wider land holdings appear to have been cut for silage and ploughed (photograph 3).

- 3.2.2 – No hedgerow improvements were observed.

- 3.3.1 – No new ponds have been created; both these areas are ploughed fields.

- 3.3.2 – No great crested newt refugia have been created.

- 3.3.3 – No hedge planting has been carried out.

- 3.3.4 – The bat roost has not been created.

- 3.3.5 – There is no barn owl box installed.

- 3.4 – Himalayan Balsam is present all along the watercourse in some dense stands.  Clearly no management has occurred there is also at least 1 stand of Japanese knotweed (photograph 4).

- 4. Monitoring – Clearly none has taken place in 2 years.

- Method Statement for Great Crested Newt, Bats, European Badger  and European Otter

3.1.1.1 – No great crested newt mitigation area has been created.

3.2.2.1 – Clearly no monitoring has taken place.