For the Attention of: Sharon Holman – Case Officer

Wrexham County Borough Council

[By Email: planning@wrexham.gov.uk]

20 June 2017

Dear Ms Holman

PLANNING APPLICATION: P/2017/0437

Application for variation of condition No. 1 imposed under planning permission P/2012/0364 to allow a further 5 years for commencement of development (erection of 4 no. detached two-storey dwellings with integral garages) at Land North of 8 Grango Lane, Ponciau, Wrexham

Thank you for your consultation email of 1 June 2017 seeking the views of The Coal Authority on the above planning application.

The Coal Authority is a non-departmental public body sponsored by the Department for Business, Energy & Industrial Strategy. As a statutory consultee, The Coal Authority has a duty to respond to planning applications and development plans in order to protect the public and the environment in mining areas.

The Coal Authority Response: Fundamental Concern

I have reviewed the proposals and confirm that the application site falls within the defined Development High Risk Area.

The Coal Authority records indicate that within the application site and surrounding area there are coal mining features and hazards which need to be considered in relation to the determination of this planning application.

Our information indicates that a recorded mine entry (shaft, CA ref. 329346-060) is present within the application site and that the zones of influence of two further mine entries (shafts, CA refs. 329346-061and 329346-529), which are present immediately adjacent to the site boundaries, encroach into the site. We hold no details of any treatment of these coal mining feature and, due to potential plotting inaccuracies, the precise position of the shafts may deviate from their plotted positions by several metres. In addition, historic unrecorded coal mining is likely to have taken place beneath the site at shallow depth.

The Coal Authority notes that the current planning application seeks to vary Condition 1 of planning permission P/2012/0364 to allow a further 5 years for the commencement of development relating to a scheme of residential development comprising four dwellings, originally approved on 2 July 2012.

As you will be aware, The Coal Authority is a statutory consultee where planning applications propose development within the Development High Risk Area. As such, we would have expected to have been consulted on planning application P/2012/0364 back in 2012. Having reviewed our records it does not appear that we were consulted by the LPA on that application.

The Coal Authority therefore objects to the current planning application, as the required Coal Mining Risk Assessment Report, or equivalent, has not been submitted as part of the application.

With reference to the mine entries, any form of development over or within the influencing distance of these features can be dangerous and raises significant safety and engineering risks and exposes all parties to potential financial liabilities. The Coal Authority has adopted a policy where, as a general precautionary principle, the building over or within the influencing distance of a mine entry should wherever possible be avoided. Our adopted policy on this matter can be found at:

www.gov.uk/government/publications/building-on-or-within-the-influencing-distance-of-mine-entries

Based on the information shown on the proposed site layout, whilst the plotted positions of the recorded mine entries are not shown, based on our records it would appear that the westernmost of the proposed dwellings would be sited over or very close to the plotted positon of mine entry 329346-060. This would clearly conflict with The Coal Authority’s aforementioned policy and we therefore object to the layout as currently proposed.

Given the risk and uncertainty regarding the coal mining legacy at the site, particularly that derived from the presence of mine entries within/adjacent to the site, further specialist investigation, subject to obtaining Coal Authority permission, is required prior to the determination of any detailed planning application to ensure that sufficient information is available for the LPA to be satisfied that the application site can be safely developed.

The results of these investigations should be used to inform any necessary revisions to the proposed layout to avoid the mine entries and their associated zones of influence. Any necessary remedial, preventive and mitigation measures should then be proposed as part of the required Coal Mining Risk Assessment Report, to address any issues of land instability.

It is a requirement of Planning Policy Wales, paragraph 13.9, that the applicant demonstrates to the satisfaction of the LPA that the application site is safe, stable and suitable for development. In addition, the national validation list in Wales requires planning applications in the defined Development High Risk Area to be accompanied by a Coal Mining Risk Assessment.

The Coal Authority Recommendation to the LPA

In accordance with the agreed risk-based approach to development management in Development High Risk Areas, the applicant should be informed that they need to submit a Coal Mining Risk Assessment Report as part of this application.

Without such an assessment of any risks to the development proposal posed by past coal mining activity, based on up-to-date coal mining information and the results of intrusive site investigations, the Coal Authority does not consider that the LPA has sufficient information to determine this planning application and therefore objects to this proposal.

If the applicant ultimately fails to demonstrate to the LPA that the application site is safe and stable to accommodate the proposed development then the LPA may refuse planning permission, in accordance with Planning Policy Wales, paragraph 13.9.

The Coal Authority would be very pleased to receive for further consultation and comment any subsequent Coal Mining Risk Assessment Report which is submitted in support of this planning application.

Please do not hesitate to contact me if you would like to discuss this matter further.

Yours sincerely

James Smith

James Smith BSc. (Hons), Dip.URP, MRTPI

Planning Liaison Manager

General Information for the Applicant

The Coal Mining Risk Assessment needs to interpret the coal mining risks and should be based on up-to-date information of past coal mining activities in relation to the application site. A variety of Coal Mining Report products which provide baseline information on coal mining legacy risks are available from www.groundstability.com. A Coal Mining Risk Assessment should then take the information contained in the Coal Mining Report and interpret the risks identified specifically in relation to the proposed development. If you merely submit a Non Residential Coal Mining Report, an Enviro All-in-One Report or other factual report obtained from www.groundstability.com (or a similar product from private land search suppliers) this will not overcome our objection to your planning application.

This coal mining information you obtain from a Non-Residential Coal Mining Report, an Enviro-All-in-One Report or other factual report should then be used to assess whether or not past mining activity poses any risk to the development proposal and, where necessary, propose mitigation measures to address any issues of land instability. This could include further intrusive investigation on site to ensure that the Local Planning Authority has sufficient information to determine the planning application.

The Coal Mining Risk Assessment should be prepared by a “competent body”. Links to the relevant professional institutions of competent bodies can be found at:

www.gov.uk/planning-applications-coal-mining-risk-assessments

Guidance on how to produce a Coal Mining Risk Assessment and a template which the “competent body” can utilise is also contained at:

www.gov.uk/planning-applications-coal-mining-risk-assessments

As one of the coal mining legacy issues that needs further consideration in this particular case is potential historic shallow mining the British Geological Survey (BGS) may prove a useful source of geological and mining information: www.bgs.ac.uk

Under the Coal Industry Act 1994 any intrusive activities, including initial site investigation boreholes, and/or any subsequent treatment of coal mine workings/coal mine entries for ground stability purposes require the prior written permission of the Coal Authority, since such activities can have serious public health and safety implications. Failure to obtain permission will result in trespass, with the potential for court action. In the event that you are proposing to undertake such work in the Forest of Dean local authority area our permission may not be required; it is recommended that you check with us prior to commencing any works. Application forms for Coal Authority permission and further guidance can be obtained from the Coal Authority’s website at:

www.gov.uk/get-a-permit-to-deal-with-a-coal-mine-on-your-property

Disclaimer

The above consultation response is provided by the Coal Authority as a Statutory Consultee and is based upon the latest available coal mining data on the date of the response, and electronic consultation records held by the Coal Authority since 1 April 2013. The comments made are also based upon only the information provided to the Coal Authority by the Local Planning Authority and/or has been published on the Council's website for consultation purposes in relation to this specific planning application. The views and conclusions contained in this response may be subject to review and amendment by the Coal Authority if additional or new data/information (such as a revised Coal Mining Risk Assessment) is provided by the Local Planning Authority or the applicant for consultation purposes.