| 4.5.2 The Old Black Horse - BS5837 Inspection - BJUFC – 7th October 2016 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. T=tree S= shrub H= hedge G= group | Species | Dbh (stem diam @ 1.5m ht) mm. | Total height. Ht to base of crown. Est Ht in 10 yrs. m. |  |  | Crown radii m. |  |  |  | Age class | Health | Structural Condition | SULE | Comment (All are in average to good health and condition, unless stated otherwise.) | Retention category A (best) to C. U = (remove) Sub-category 1, 2 or 3 | BS 5837 Root Protection Area.m. | Recommended WORK excluding development. |
|  |  |  |  |  |  | NNW | ENE | ESE | WNW |  |  |  |  |  |  |  |  |
| T1 | Ash | 310 | 11 | 4 | 14 | 5 | 4 | 0 | 3 | Sm | F | F | 20+ | Off-site. Self-sown and pushing wall. | B2 | 3.7 |  |
| T2 | Hazel | 40 stems x 140 | 8 | 1 | 8 | 6 | 6 | 6 | 5 | Lm | F | F | 20-40 | Off-site. Pushing wall. HUGE. | A3 Veteran | 10.6 ** | ** RPA may be exaggerated on this large shrub / small tree. |
| H3 | Privet | 100 | 2 | 0 | 2 | 0.5 | 0.5 | 0.5 | 0.5 | Em | P/F | F | 20+ | Hedge on boundary, suppressed under hazel. | C2 | 1.2 |  |
| T4 | Goat willow | 60, 60 | 4 | 1 | 0 | 1 | 1 | 1 | 1 | Y | F | F | <5 | Self-sown by fence. No space to groww. | U |  | Fell and poison stump. |
| H5A | Beech & silver birch | 150 | 6 | 0 | 9 | 1.2 | 1.2 | 1.2 | 1.2 | Y | F | F | >40 | Off-site beech hedge with eleven silver birch. | C2 | 1.8 | Advise owner to crown lift silver birch’s to half height above hedge. Top beech hedge at 2m. |
| H5B | Beech & wild cherry | 150 | 4-5 | 0 | 5-7 | 1 | 1 | 1 | 1 | Y | F | F | 20-40 | Beech hedge with about ten wild cherries. | C2 | 1.8 | Advise owner to crown lift wild cherries to half height above hedge. Top beech hedge at 2m. |
| H5C | Beech | 100 | 2 | 0 | 2 | 0.5 | 0.5 | 0.5 | 0.5 | Y | P/F | P/F | >40 | Off-site beech hedge. East of 9m gap. | C2 | 1.2 | Advise owner to infill plant 9m gap between H5B and H5C. |
| G6 | Hazels x 2 | 5 stems avg 70 | 4 | 0 | 5.5 | 1.5 | 1.75 | 2 | 1.5 | Sm / Em | F | F | 20-40 | Just inside boundary. | C2 | 1.9 |  |
| T7 | Ash | 330, 710 | 12 | 2 | 14 | 8 | 7 | 6 | 7 | Em | F | P/F | 20-40 | Broad. Self-sown in corner of pub garden. Stem to south cut off, leaving a slight hole in the canopy. | B1 | 9.4 |  |
| T8 | Flowering cherry | 350 | 9 | 2.5 | 10 | 5 | 4 | 5 | 5 | M | F | F | 10-20 | Off-site in school. Part of a group of broadleaves in the playing field corner. | C1 | 4.2 |  |
| T9 | Silver birch | Est 250 | 9 | 2 | 12 | 4 | 4 | 2 | 3 | Em | F | F | >40 | Off-site in school. Part of a group of broadleaves in the playing field corner. | B1 | 3.0 |  |
| T10 | Silver birch | Est 250 | 9 | 2 | 12 | 3 | 5 | 2 | 1.5 | Em | F | F | >40 | Off-site in school. Part of a group of broadleaves in the playing field corner. | B2 | 3.0 |  |

End of table.